| # FEATURES |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  Meet IEEE 802.3 and ANSI X3.263 standards including 350 uH OCL with 8mA Bias  Desingned to withstand IR reflow temperatures up to 235∘C  Compact footprint use in the most space restrictive applications  Recognized by UL 1950. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| # ELECTRICAL SPECIFICATIONS @25℃-Operating temperature -40℃ TO 85℃ |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Part Number | Turns Ratio (±5%) |  | Insertion Loss (dB MAX) | Return Loss (dB TYP) |  |  | Differential to Common Mode Rejection (dB MIN) |  |  |  | Crosstalk (dB MIN) |  |  | Hipot VrmS MIN |
|  | TX | RX | 0.1-100MHz | 30 MHz | 60 MHz | 80 MHz | 30 MHz |  | 50 MHz | 100 MHz | 30 MHz | 60 MHz | 100 MHz |  |
| 11F-05 | 1:1 | 1:1 | -1.1 | -20 | -14 | -11.5 | -42 |  | -37 | -33 | -45 | -40 | -35 | 1500 |
| 11F-06 | 1.25:1 | 1:1 | -1.1 | -20 | -14 | -11.5 | -42 |  | -37 | -33 | -45 | -40 | -35 | 1500 |
| 11F-07 | 2:1 | 1:1 | -1.1 | -20 | -14 | -11.5 | -42 |  | -37 | -33 | -45 | -40 | -35 | 1500 |
| 11F-08 | 1:1.414 | 1:1 | -1.1 | -18 | -12 | -11.5 | -42 |  | -37 | -30 | -45 | -40 | -35 | 1500 |
| 11F-09 | 1:1 | 1:1 | -1.0 | -18 | -14 | -12 | -40 |  | -40 | -40 | -35 | -35 | -35 | 1500 |
| 11F-10 | 1:1 | 1:1 | -0.9 | -18 | -12 | -11 | -40 |  | -35 | -30 | -45 | -45 | -37 | 1500 |
| 11F-11 | 1:1 | 1:1 | -1.0 | -20 | -14 | -12 | -42 |  | -37 | -32 | -45 | -40 | -35 | 1500 |
| 11F-12 | 1:1 | 1:1 | -1.0 | -20 | -14 | -12 | -42 |  | -37 | -32 | -45 | -40 | -35 | 1500 |
| MECHANICAL |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| SCHEMATICS |  |
| --- | --- |
| 11F-05, 06, 07 08, 11 |  |
| 11F-10 |  |
| 11F-09, 12 |  |